# IOT ENABLED SMART FARMING

# APPLICATION

# CODING :

#include <Servo.h>

int output1Value = 0; int sen1Value = 0;

int sen2Value = 0;

int const gas_sensor = A1; int const LDR = A0;

int limit = 400;

long readUltrasonicDistance(int triggerPin, int echoPin)

{

pinMode(triggerPin, OUTPUT); // Clear the trigger digitalWrite(triggerPin, LOW); delayMicroseconds(2);

// Sets the trigger pin to HIGH state for 10 microseconds digitalWrite(triggerPin, HIGH);

delayMicroseconds(10); digitalWrite(triggerPin, LOW); pinMode(echoPin, INPUT);

// Reads the echo pin, and returns the sound wave travel time in microseconds return pulseIn(echoPin, HIGH);

}

Servo servo_7;

void setup()

{

Serial.begin(9600);	//initialize serial communication pinMode(A0, INPUT);	//LDR

pinMode(A1,INPUT);	//gas sensor

pinMode(13, OUTPUT);	//connected to relay servo_7.attach(7, 500, 2500); //servo motor

pinMode(8,OUTPUT);	//signal to piezo buzzer pinMode(9, INPUT);	//signal to PIR

pinMode(10, OUTPUT);		//signal to npn as switch pinMode(4, OUTPUT);	//Red LED

pinMode(3, OUTPUT);	//Green LED

}

void loop()

{

//------light intensity control	//

//-

int val1 = analogRead(LDR); if (val1 > 500)

{

digitalWrite(13, LOW); Serial.print("Bulb ON = ");

Serial.print(val1);

}

else

{

digitalWrite(13, HIGH); Serial.print("Bulb OFF = "); Serial.print(val1);

}

//-

//------ light & fan control	//

//-

sen2Value = digitalRead(9); if (sen2Value == 0)

{

digitalWrite(10, LOW); //npn as switch OFF

digitalWrite(4, HIGH); // Red LED ON,indicating no motion digitalWrite(3, LOW); //Green LED OFF, since no Motion detected

Serial.print("	|| NO Motion Detected " );

}

if (sen2Value == 1)

{

digitalWrite(10, HIGH);//npn as switch ON delay(5000);

digitalWrite(4, LOW); // RED LED OFF

digitalWrite(3, HIGH);//GREEN LED ON , indicating motion detected Serial.print("	|| Motion Detected!	" );

//-	-

// ------- Gas Sensor	//

//-	-

int val = analogRead(gas_sensor);	//read sensor value Serial.print("|| Gas Sensor Value = ");

Serial.print(val);	//Printing in serial monitor

//val = map(val, 300, 750, 0, 100); if (val > limit)

{

tone(8, 650);

}

delay(300); noTone(8);

//-

//------- servo motor	//

//-	- sen1Value = 0.01723 * readUltrasonicDistance(6, 6);

if (sen1Value < 100)

{

servo_7.write(90);

Serial.print("	|| Door Open! ; Distance = "); Serial.print(sen1Value);

Serial.print("\n");

else

{

servo_7.write(0);

Serial.print("	|| Door Closed! ; Distance = "); Serial.print(sen1Value);

Serial.print("\n");

}

delay(10); // Delay a little bit to improve simulation performance

}

# IMAGE :